PRG_CAM · PROJECT REVIEW & TRAINING SERIES

05

Python 接收机架构与演进

从 Npcap Ethernet frame 到 COMPLETE / RECOVERED 图像：协议、会话、队列和存储

| 适用对象 | 具备基础 Verilog/Python，正在学习 Vivado、Ethernet 与端到端调试的工程人员 |
| --- | --- |
| 范围 | CLI、Layer1–5、Camera协议、重组、恢复、异步输出、Storage/Audit/Monitor与pytest |
| 当前状态 | 自动测试：PASS（88项）；优化后的正式120秒live验收：PENDING |
| 事实基线 | 当前源码/.xpr > 当前报告/测试/PCAP/CSV > Git > 技术文档 > 历史推测 |
| 来源重组 | 本主题无一一对应旧Word；根据当前Python源码、Git diff、p11、attempt3和技术文档重建 |

阅读约束：本文把 PASS、PARTIAL、PENDING 分开。实现时序通过不等于板级通过；ILA 只增加可观察性；LAST_ROW 只表示看见帧尾标志，不等于 480 行完整。

版本：Phase 1 training set · 2026-07-28

目录

在 Word 中打开后更新目录

提示：本目录为 Word 字段。若页码未自动刷新，右键目录选择“更新域 → 更新整个目录”。

# 1. 事实边界与软件入口

本章目标｜先区分代码测试通过、离线PCAP通过和真实live性能通过。

表 1-1  当前Python状态

| 项目 | 状态 | 证据/边界 |
| --- | --- | --- |
| 全量pytest | PASS | 88 passed in 5.20s |
| Fixed PCAP Layer1/2 | PASS | 1000帧及229629帧00..7F |
| Layer3协议/CRC/flags | PASS（测试） | packet_format/camera_parser |
| 跨帧隔离/zero-fill | PASS（测试） | reassembler/image recovery tests |
| attempt3输出一致性 | PASS（有限） | PGM/RAW mismatch=0；zero-fill failure=0 |
| attempt3性能 | FAIL/PARTIAL | 82,135 capture queue drops；usable 8.453fps |
| 性能优化后120秒 | PENDING | 尚无同条件final report |

重要口径｜packet rate不是image fps；LAST_ROW次数不是完整图数量；RECOVERED是输出策略，不是Reassembler的FrameStatus枚举。

# 2. Layer1–5、线程与两类有界队列

图 2-1  Python接收机的当前真实调用链

表 2-1  层级与职责

| 阶段 | 主要文件 | 输入→输出 | 失败行为 |
| --- | --- | --- | --- |
| Layer1 | capture.py | NIC/PCAP→RawEthernetFrame | 权限/接口/捕获错误 |
| Capture queue | pipeline.py | frame对象→packet worker | live满则drop |
| Layer2 | eth_validate.py | Ethernet→payload | MAC/type/length失败 |
| Layer3 | camera_parser.py | 128B→CameraPacket | 带reason严格拒绝 |
| Layer4 | stream_monitor.py | packet→统计 | gap/duplicate/out-of-order |
| Layer5 | reassembler.py | row packets→session | partial/corrupt/timeout |
| Output queue | async_sink.py | CompletedFrame→worker | 满时阻塞 |
| Storage/Image | storage.py/image_pipeline.py | frame→RAW/PGM/JSON/CSV | 原子写/失败计数 |

图 2-1 中两个queue不能混为一谈：Capture queue的数据单位是Ethernet包；Frame output queue的数据单位是已关闭图像会话。

# 3. 128-byte Camera payload

图 3-1  Camera header、packed row、trailer与CRC16边界

表 3-1  当前payload逐字节字段

| Offset | 长度 | 字段 | 端序/语义 |
| --- | --- | --- | --- |
| 0 | 2 | sync0 | 0xA5A0，线上A5 A0 |
| 2 | 2 | sync1 | 0x5A50，线上5A 50 |
| 4 | 1 | cam_id | 0..3 |
| 5 | 2 | frame_id | 大端 |
| 7 | 2 | row_idx | 大端，0..479 |
| 9 | 1 | row_flags | 0x01 overflow/0x02 last/0x04 first/0x08 length |
| 10 | 1 | payload_len | 有效packed row bytes |
| 11 | 2 | row_seq | 大端，16-bit连续计数 |
| 13 | 11 | reserved | 当前保留 |
| 24 | 80 | row payload | 640像素1bpp packed |
| 104 | 10 | trailer pad | 应为0 |
| 114 | 12 | m00/xc/yc/vx/vy | 大端，vx/vy signed |
| 126 | 2 | CRC16 | 小端，覆盖0..125 |

代码对照｜协议常量、flag和struct

0028 ¦ HEADER_LEN = 24 0029 ¦ TRAILER_LEN = 24 0030 ¦ PACKET_LEN = 128 0031 ¦ ROW_BYTES = PACKET_LEN - HEADER_LEN - TRAILER_LEN # 80 0032 ¦ 0033 ¦ # Current RP2350A wire semantics, confirmed against the live attempt2 CSV: 0034 ¦ # bit 0 reports a frame/line-buffer overflow, bit 1 terminates the frame, and 0035 ¦ # bit 2 marks the first valid row. Older project-side code had bit 0 and bit 2 0036 ¦ # swapped; that made every real 0x04 first-row packet look corrupt. 0037 ¦ FLAG_FRAME_OVERFLOW = 1 << 0 0038 ¦ FLAG_LAST_ROW = 1 << 1 0039 ¦ FLAG_FIRST_ROW = 1 << 2 0040 ¦ FLAG_LENGTH_ERROR = 1 << 3 0041 ¦ 0042 ¦ # Protocol word values and their expected MSB-byte-first wire representation. 0043 ¦ # This is byte order inside each multi-byte metadata field; it is not an 0044 ¦ # MSB/LSB bit reversal inside an individual byte. 0045 ¦ SYNC0_DEFAULT = 0xA5A0 0046 ¦ SYNC1_DEFAULT = 0x5A50 0047 ¦ SYNC_BYTES_DEFAULT = struct.pack(">HH", SYNC0_DEFAULT, SYNC1_DEFAULT) 0048 ¦ assert SYNC_BYTES_DEFAULT == bytes.fromhex("a5a05a50") 0049 ¦ 0050 ¦ _LEGACY_SYNC_BYTES = bytes.fromhex("a5a55a5a") 0051 ¦ 0052 ¦ # ---- pkt_row_header_t --------------------------------------------------- 0053 ¦ # uint16 sync0, uint16 sync1, uint8 cam_id, uint16 frame_id, uint16 row_idx, 0054 ¦ # uint8 row_flags, uint8 payload_len, uint16 row_seq, uint8 reserved[11] 0055 ¦ _HEADER_STRUCT_BE = struct.Struct(">HHBHHBBH11s") 0056 ¦ _HEADER_STRUCT_LE = struct.Struct("<HHBHHBBH11s") 0057 ¦ assert _HEADER_STRUCT_BE.size == HEADER_LEN 0058 ¦ assert _HEADER_STRUCT_LE.size == HEADER_LEN 0059 ¦ 0060 ¦ # ---- plt_row_trailer_t --------------------------------------------------- 0061 ¦ # uint8 pad[10], uint32 m00, uint16 xc_q4, uint16 yc_q4, 0062 ¦ # int16 vx_q8, int16 vy_q8, uint16 crc16 0063 ¦ _TRAILER_BODY_STRUCT_BE = struct.Struct(">10sIHHhh") 0064 ¦ _TRAILER_BODY_STRUCT_LE = struct.Struct("<10sIHHhh") 0065 ¦ assert _TRAILER_BODY_STRUCT_BE.size == TRAILER_LEN - 2 0066 ¦ assert _TRAILER_BODY_STRUCT_LE.size == TRAILER_LEN - 2 0067 ¦ 0068 ¦ # Everything except the trailing crc16 -- this is exactly what the CRC 0069 ¦ # is calculated over (mirrors the original code's payload[:126]). 0070 ¦ _CRC_COVERED_LEN = PACKET_LEN - 2 0071 ¦ _BODY_STRUCT_BE = struct.Struct(f">HHBHHBBH11s{ROW_BYTES}s10sIHHhh") 0072 ¦ _BODY_STRUCT_LE = struct.Struct(f"<HHBHHBBH11s{ROW_BYTES}s10sIHHhh") 0073 ¦ assert _BODY_STRUCT_BE.size == _CRC_COVERED_LEN 0074 ¦ assert _BODY_STRUCT_LE.size == _CRC_COVERED_LEN

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\packet_format.py:28

0x58不是新flag｜合法flag集合只定义低4位。attempt3中的0x58与字段错位/短行相符，不能通过“新增0x58定义”来掩盖。

# 4. Layer1 Capture 与 Layer2 Ethernet Decode

代码对照｜Npcap/Scapy BPF与按需raw frame复制

0071 ¦ def start(self, on_frame: Callable[[RawEthernetFrame], None]) -> None: 0072 ¦ from scapy.all import AsyncSniffer 0073 ¦ from scapy.layers.l2 import Ether 0074 ¦ 0075 ¦ def _callback(packet) -> None: 0076 ¦ if Ether not in packet: 0077 ¦ return 0078 ¦ eth = packet[Ether] 0079 ¦ on_frame(RawEthernetFrame( 0080 ¦ src_mac=eth.src, 0081 ¦ dst_mac=eth.dst, 0082 ¦ ethertype=int(eth.type), 0083 ¦ payload=bytes(eth.payload), 0084 ¦ # The complete second byte copy is needed only when the 0085 ¦ # optional PCAP recorder is enabled. 0086 ¦ raw_bytes=bytes(packet) if self.include_raw else b"", 0087 ¦ timestamp=time.time(), 0088 ¦ )) 0089 ¦ 0090 ¦ self._sniffer = AsyncSniffer( 0091 ¦ iface=self.interface, 0092 ¦ filter=f"ether proto 0x{self.ether_type:04x}", 0093 ¦ prn=_callback, 0094 ¦ store=False, 0095 ¦ ) 0096 ¦ self._sniffer.start()

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\capture.py:71

BPF在capture层使用ether proto 0x88b5，降低无关帧进入Python。

payload始终转成独立bytes；只有启用PCAP录制时才复制整个bytes(packet)。

因此取消第二次完整frame复制后，减少CPU、内存带宽和对象分配，同时不依赖Scapy底层buffer生命周期。

Layer2校验dst/src/type/最小长度；Wireshark灰色duplicate不是自动坏帧判据。

代码对照｜Capture queue的live drop与offline背压

0134 ¦ # ---- Layer 1 -> queue hand-off (runs on the capture thread) ------- 0135 ¦ 0136 ¦ def _on_frame(self, frame: RawEthernetFrame) -> None: 0137 ¦ self.monitor.record_ethernet_frame() 0138 ¦ if self.lossless_input: 0139 ¦ while not self._stop_event.is_set(): 0140 ¦ try: 0141 ¦ self._queue.put(frame, timeout=0.2) 0142 ¦ self.monitor.record_capture_queue_depth( 0143 ¦ self._queue.qsize() 0144 ¦ ) 0145 ¦ return 0146 ¦ except queue.Full: 0147 ¦ continue 0148 ¦ return 0149 ¦ try: 0150 ¦ self._queue.put_nowait(frame) 0151 ¦ self.monitor.record_capture_queue_depth(self._queue.qsize()) 0152 ¦ except queue.Full: 0153 ¦ self.monitor.record_dropped_capture() 0154 ¦ 0155 ¦ # ---- worker thread: run every frame through the stage chain ------- 0156 ¦ 0157 ¦ def _run_worker(self) -> None: 0158 ¦ while not self._stop_event.is_set() or not self._queue.empty(): 0159 ¦ try: 0160 ¦ frame = self._queue.get(timeout=0.2) 0161 ¦ except queue.Empty: 0162 ¦ for stage in self._reassembly_stages: 0163 ¦ stage.poll_timeouts() 0164 ¦ self.monitor.maybe_report() 0165 ¦ continue 0166 ¦ 0167 ¦ try: 0168 ¦ self._process_frame(frame) 0169 ¦ except Exception as exc: # noqa: BLE001 -- keep the worker alive 0170 ¦ # This boundary includes observer/storage callbacks as well as 0171 ¦ # parsing stages. Calling every such exception a parser error 0172 ¦ # made WinError 5 archive failures look like bad packet data. 0173 ¦ self.monitor.stats.processing_errors += 1 0174 ¦ print(f"[PROCESSING ERROR] {exc}", file=sys.stderr) 0175 ¦ finally: 0176 ¦ self._queue.task_done()

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\pipeline.py:134

# 5. Layer3 严格解析

代码对照｜sync、payload_len、CRC和字段错误分类

0062 ¦ def parse_camera_mode(payload: bytes) -> CameraModeResult: 0063 ¦ if len(payload) != PACKET_LEN: 0064 ¦ return CameraModeResult( 0065 ¦ ok=False, 0066 ¦ reason="bad_length", 0067 ¦ received_len=len(payload), 0068 ¦ errors=("bad_length",), 0069 ¦ ) 0070 ¦ 0071 ¦ packet = parse_camera_row(payload) 0072 ¦ errors: list[str] = [] 0073 ¦ warnings: list[str] = [] 0074 ¦ 0075 ¦ # Current protocol words are 0xA5A0/0x5A50 and the observed/expected raw 0076 ¦ # Ethernet payload bytes are A5 A0 5A 50 (MSB byte first). There is no 0077 ¦ # protocol-version field; reject another pair instead of silently parsing 0078 ¦ # an incompatible layout. 0079 ¦ if ( 0080 ¦ packet.header.sync0 != SYNC0_DEFAULT 0081 ¦ or packet.header.sync1 != SYNC1_DEFAULT 0082 ¦ ): 0083 ¦ errors.append("bad_sync") 0084 ¦ 0085 ¦ # payload_len describes valid bytes inside the fixed 80-byte row payload. 0086 ¦ # Zero is not rejected because the RP2350A source definition is absent; 0087 ¦ # surface it as a warning until firmware defines whether an empty row is 0088 ¦ # legal. A value larger than the physical field is unambiguously invalid. 0089 ¦ if packet.header.payload_len > ROW_BYTES: 0090 ¦ errors.append("payload_len_out_of_range") 0091 ¦ elif packet.header.payload_len == 0: 0092 ¦ warnings.append("zero_payload_len") 0093 ¦ 0094 ¦ if not packet.crc_ok: 0095 ¦ errors.append("crc_error") 0096 ¦ 0097 ¦ # These flags are generated/ORed by the active FPGA capture path and make 0098 ¦ # the packet unsuitable for image reconstruction even when its post-capture 0099 ¦ # CRC is correct. 0100 ¦ if packet.frame_overflow: 0101 ¦ errors.append("frame_overflow") 0102 ¦ if packet.length_error: 0103 ¦ errors.append("length_error") 0104 ¦ 0105 ¦ return CameraModeResult( 0106 ¦ ok=not errors, 0107 ¦ reason=errors[0] if errors else "", 0108 ¦ packet=packet, 0109 ¦ received_len=len(payload), 0110 ¦ errors=tuple(errors), 0111 ¦ warnings=tuple(warnings), 0112 ¦ )

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\camera_parser.py:62

表 5-1  Layer3原则

| 检查 | 失败结果 | 为什么不放宽 |
| --- | --- | --- |
| 固定128-byte长度 | bad length | 否则字段offset不可相信 |
| A5A0/5A50 sync | bad_sync | 防止错位数据伪装成header |
| payload_len≤80 | out_of_range | 防止越过行payload |
| CRC16 | crc_error | Camera层完整性 |
| 保留/flag语义 | warning/error reason | 保留原始证据 |

Layer3失败包不会进入frame.rows。恢复模式也不会使用其payload；RECOVERED只对missing row_idx写一整行0。

# 6. Layer4 Monitor：统计与速率口径

代码对照｜producer/consumer、queue peak和最终报告

0196 ¦ def final_report(self) -> None: 0197 ¦ elapsed = time.monotonic() - self.stats.start_time 0198 ¦ average_packets_per_second = ( 0199 ¦ self.stats.matching_frames / elapsed if elapsed > 0 else 0.0 0200 ¦ ) 0201 ¦ producer_rate = ( 0202 ¦ self.stats.total_ethernet_frames / elapsed 0203 ¦ if elapsed > 0 0204 ¦ else 0.0 0205 ¦ ) 0206 ¦ queue_drop_percent = ( 0207 ¦ self.stats.dropped_capture_queue 0208 ¦ * 100.0 0209 ¦ / self.stats.total_ethernet_frames 0210 ¦ if self.stats.total_ethernet_frames 0211 ¦ else 0.0 0212 ¦ ) 0213 ¦ average_mbps = ( 0214 ¦ self.stats.total_payload_bytes * 8 / elapsed / 1_000_000 if elapsed > 0 else 0.0 0215 ¦ ) 0216 ¦ 0217 ¦ self.sink("\n============== FINAL REPORT ==============") 0218 ¦ self.sink(f"Elapsed : {elapsed:.3f} s") 0219 ¦ self.sink(f"Capture ingress : {self.stats.total_ethernet_frames}") 0220 ¦ self.sink(f"Matching Ethernet : {self.stats.matching_frames}") 0221 ¦ self.sink(f"Valid packets : {self.stats.valid_packets}") 0222 ¦ self.sink(f"Bad Ethernet length : {self.stats.bad_ethernet_length}") 0223 ¦ self.sink(f"Bad fixed payload : {self.stats.bad_fixed_payload}") 0224 ¦ self.sink(f"CRC errors : {self.stats.bad_crc}") 0225 ¦ self.sink(f"Parser errors : {self.stats.parser_errors}") 0226 ¦ self.sink(f"Processing errors : {self.stats.processing_errors}") 0227 ¦ self.sink(f"Capture queue drops : {self.stats.dropped_capture_queue}") 0228 ¦ self.sink( 0229 ¦ f"Capture queue peak : " 0230 ¦ f"{self.stats.capture_queue_peak}/" 0231 ¦ f"{self.stats.capture_queue_capacity}" 0232 ¦ ) 0233 ¦ self.sink(f"Capture drop percent : {queue_drop_percent:.4f}%") 0234 ¦ self.sink(f"Producer rate : {producer_rate:.2f} packets/s") 0235 ¦ self.sink( 0236 ¦ f"Consumer rate : " 0237 ¦ f"{average_packets_per_second:.2f} packets/s" 0238 ¦ ) 0239 ¦ self.sink( 0240 ¦ f"Average packet rate : " 0241 ¦ f"{average_packets_per_second:.2f} packets/s" 0242 ¦ ) 0243 ¦ self.sink(f"Average payload rate : {average_mbps:.3f} Mb/s")

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\stream_monitor.py:196

表 6-1  监控指标如何解释

| 指标 | 单位/起点 | 增长说明 |
| --- | --- | --- |
| producer_rate | matching Ethernet包/s | capture回调向队列提供速度 |
| consumer_rate | 完成stage链的包/s | packet worker实际消费速度 |
| queue_peak/capacity | 对象数 | 接近容量说明突发或长期积压 |
| queue_drops | live队列满后主动丢弃包数 | Python接收链无法跟上 |
| complete/recovered fps | 图像/s | 与packet rate完全不同 |
| output failures | 完成帧输出失败次数 | 磁盘/worker异常 |

# 7. Layer5 FrameReassembler

图 7-1  以(cam_id, frame_id)隔离的会话状态

代码对照｜session key、frame switch与严格row接收

0176 ¦ cam_id = packet.header.cam_id 0177 ¦ key = (cam_id, packet.header.frame_id) 0178 ¦ 0179 ¦ # A new frame ID for one camera closes that camera's older sessions, 0180 ¦ # but never touches another camera's state. Equality is used instead 0181 ¦ # of ordering so the 16-bit frame ID can wrap from 65535 to 0. 0182 ¦ for old_frame_id in list(self._order.get(cam_id, [])): 0183 ¦ if old_frame_id != packet.header.frame_id: 0184 ¦ closed = self._close( 0185 ¦ (cam_id, old_frame_id), 0186 ¦ close_reason="frame_switch", 0187 ¦ ended_at=event_time, 0188 ¦ ) 0189 ¦ if closed is not None: 0190 ¦ self._pending.append(closed) 0191 ¦ 0192 ¦ if key not in self._open: 0193 ¦ self._evict_if_needed(cam_id, event_time) 0194 ¦ self._open[key] = _FrameSession( 0195 ¦ camera_id=cam_id, 0196 ¦ frame_id=packet.header.frame_id, 0197 ¦ started_at=event_time, 0198 ¦ last_activity=event_time, 0199 ¦ ) 0200 ¦ self._order.setdefault(cam_id, []).append(packet.header.frame_id) 0201 ¦ self.stats.sessions_created += 1 0202 ¦ 0203 ¦ session = self._open[key] 0204 ¦ session.last_activity = event_time 0205 ¦ session.saw_first ¦= packet.first_row 0206 ¦ session.saw_last ¦= packet.last_row 0207 ¦ if packet.last_row: 0208 ¦ session.last_row_idx = packet.header.row_idx 0209 ¦ session.had_overflow ¦= packet.frame_overflow 0210 ¦ 0211 ¦ row_idx = packet.header.row_idx 0212 ¦ payload = packet.payload[: packet.header.payload_len] 0213 ¦ duplicate = row_idx in session.rows 0214 ¦ conflict = duplicate and session.rows[row_idx] != payload 0215 ¦ accepted = not errors and not duplicate 0216 ¦ if accepted: 0217 ¦ self.stats.rows_accepted += 1 0218 ¦ else: 0219 ¦ self.stats.rows_rejected += 1

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\reassembler.py:176

代码对照｜missing_rows、status与不可变快照

0325 ¦ if self._configured_expected_rows is not None: 0326 ¦ expected = self._configured_expected_rows 0327 ¦ elif session.last_row_idx is not None: 0328 ¦ expected = session.last_row_idx + 1 0329 ¦ else: 0330 ¦ expected = max(session.rows, default=-1) + 1 0331 ¦ missing = [i for i in range(expected) if i not in session.rows] 0332 ¦ 0333 ¦ corrupt = bool(session.errors) or session.had_overflow 0334 ¦ if corrupt: 0335 ¦ status = FrameStatus.CORRUPT 0336 ¦ elif close_reason == "timeout": 0337 ¦ status = FrameStatus.TIMEOUT 0338 ¦ elif session.saw_last and not missing: 0339 ¦ status = FrameStatus.COMPLETE 0340 ¦ else: 0341 ¦ status = FrameStatus.PARTIAL 0342 ¦ 0343 ¦ completed = CompletedFrame( 0344 ¦ camera_id=session.camera_id, 0345 ¦ frame_id=session.frame_id, 0346 ¦ row_count=len(session.rows), 0347 ¦ rows=dict(session.rows), 0348 ¦ missing_rows=missing, 0349 ¦ had_overflow=session.had_overflow, 0350 ¦ status=status, 0351 ¦ close_reason=close_reason, 0352 ¦ expected_rows=expected, 0353 ¦ packet_records=list(session.packet_records), 0354 ¦ errors=list(session.errors), 0355 ¦ duplicate_packets=session.duplicate_packets, 0356 ¦ conflicting_duplicates=session.conflicting_duplicates, 0357 ¦ started_at=session.started_at, 0358 ¦ ended_at=ended_at, 0359 ¦ saw_first=session.saw_first, 0360 ¦ saw_last=session.saw_last, 0361 ¦ ) 0362 ¦ if status is FrameStatus.COMPLETE: 0363 ¦ self.stats.frames_completed += 1 0364 ¦ elif status is FrameStatus.TIMEOUT: 0365 ¦ self.stats.frames_timed_out += 1 0366 ¦ elif status is FrameStatus.CORRUPT: 0367 ¦ self.stats.frames_corrupt += 1 0368 ¦ else: 0369 ¦ self.stats.frames_partial += 1 0370 ¦ return completed

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\reassembler.py:325

session key为(cam_id, frame_id)，不同camera不会互相覆盖。

行按row_idx写入dict，不依赖到达顺序；相同duplicate保留第一份。

同row_idx不同payload标conflicting duplicate并使会话CORRUPT。

新frame、timeout、flush或可靠LAST_ROW关闭会话；关闭时复制rows和list。

静止画面相似不能证明跨帧污染，必须比较raw row hash + missing_rows + frame_id。

# 8. COMPLETE / RECOVERED / REJECTED

图 8-1  严格帧与zero-fill恢复的准入ASM

代码对照｜COMPLETE与RECOVERED发布分支

0262 ¦ def archive_frame(self, frame: CompletedFrame) -> Path ¦ None: 0263 ¦ """Publish a COMPLETE image or an explicitly eligible RECOVERED image.""" 0264 ¦ 0265 ¦ decision: RecoveryDecision ¦ None = None 0266 ¦ if frame.status is FrameStatus.COMPLETE: 0267 ¦ self.stats.completed_frames_seen += 1 0268 ¦ output_status = "COMPLETE" 0269 ¦ missing_policy = MissingRowPolicy.REJECT 0270 ¦ elif self.image_policy is ImagePolicy.RECOVER_ZERO_FILL: 0271 ¦ self.stats.noncomplete_frames_skipped += 1 0272 ¦ decision = self._assess_recovery(frame) 0273 ¦ if not decision.eligible: 0274 ¦ self._record_rejection(frame, decision) 0275 ¦ return None 0276 ¦ output_status = "RECOVERED" 0277 ¦ missing_policy = MissingRowPolicy.ZERO_FILL 0278 ¦ else: 0279 ¦ self.stats.noncomplete_frames_skipped += 1 0280 ¦ # Preserve strict mode's pre-change disk behavior: incomplete 0281 ¦ # frames produce no image-side artifact. They are counted for 0282 ¦ # truthful rate reporting but rejected.csv is recovery-mode 0283 ¦ # evidence only. 0284 ¦ self.stats.images_rejected += 1 0285 ¦ assert self.stats.reject_reasons is not None 0286 ¦ self.stats.reject_reasons["strict_requires_complete"] += 1 0287 ¦ self._maybe_report_rates() 0288 ¦ return None 0289 ¦ 0290 ¦ try: 0291 ¦ recovered = recover_completed_frame( 0292 ¦ frame, 0293 ¦ self.expected_rows, 0294 ¦ bit_order=self.bit_order, 0295 ¦ missing_policy=missing_policy, 0296 ¦ ) 0297 ¦ except Exception: 0298 ¦ self.stats.recovery_failures += 1 0299 ¦ raise 0300 ¦ 0301 ¦ required_height = ( 0302 ¦ 480 if output_status == "RECOVERED" else self.expected_rows 0303 ¦ ) 0304 ¦ if ( 0305 ¦ recovered.height != required_height 0306 ¦ or recovered.width != ROW_PIXELS 0307 ¦ or len(recovered.pixels) != required_height * ROW_PIXELS 0308 ¦ ): 0309 ¦ raise ValueError( 0310 ¦ "image geometry mismatch after publication recovery: " 0311 ¦ f"{recovered.width}x{recovered.height}, " 0312 ¦ f"{len(recovered.pixels)} bytes" 0313 ¦ )

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\image_pipeline.py:262

代码对照｜recover-zero-fill门槛与missing_rows计算

0413 ¦ def _assess_recovery(self, frame: CompletedFrame) -> RecoveryDecision: 0414 ¦ """Apply the opt-in recovery gate without relaxing packet validation.""" 0415 ¦ 0416 ¦ reasons: list[str] = [] 0417 ¦ if self.expected_rows != 480 or frame.expected_rows != 480: 0418 ¦ reasons.append("expected_rows_not_480") 0419 ¦ if not 0 <= frame.frame_id <= 0xFFFF: 0420 ¦ reasons.append("frame_id_out_of_range") 0421 ¦ 0422 ¦ accepted_records = [ 0423 ¦ record for record in frame.packet_records if record.accepted 0424 ¦ ] 0425 ¦ reliable_last = any( 0426 ¦ record.row_idx == 479 0427 ¦ and bool(record.row_flags & FLAG_LAST_ROW) 0428 ¦ for record in accepted_records 0429 ¦ ) 0430 ¦ if not reliable_last: 0431 ¦ reasons.append("reliable_last_row_not_seen") 0432 ¦ 0433 ¦ all_errors = { 0434 ¦ error 0435 ¦ for record in frame.packet_records 0436 ¦ for error in record.errors 0437 ¦ } 0438 ¦ if frame.had_overflow or "frame_overflow" in all_errors: 0439 ¦ reasons.append("overflow") 0440 ¦ if "bad_sync" in all_errors: 0441 ¦ reasons.append("bad_sync") 0442 ¦ if "crc_error" in all_errors: 0443 ¦ reasons.append("crc_error") 0444 ¦ if frame.conflicting_duplicates or any( 0445 ¦ record.conflicting_duplicate for record in frame.packet_records 0446 ¦ ): 0447 ¦ reasons.append("conflicting_duplicate") 0448 ¦ 0449 ¦ # Length-invalid rows may account for a missing row, but their payload 0450 ¦ # is never used. Any other Layer-3 error is a hard recovery rejection. 0451 ¦ permitted_missing_row_errors = { 0452 ¦ "length_error", 0453 ¦ "payload_len_out_of_range", 0454 ¦ } 0455 ¦ unhandled_errors = sorted(all_errors - permitted_missing_row_errors - { 0456 ¦ "frame_overflow", 0457 ¦ "bad_sync", 0458 ¦ "crc_error", 0459 ¦ }) 0460 ¦ reasons.extend(f"layer3_error:{error}" for error in unhandled_errors) 0461 ¦ 0462 ¦ valid_row_indices = set(frame.rows) 0463 ¦ accepted_row_indices = {record.row_idx for record in accepted_records} 0464 ¦ if any(index < 0 or index >= 480 for index in valid_row_indices): 0465 ¦ reasons.append("row_idx_out_of_range") 0466 ¦ if any(index < 0 or index >= 480 for index in accepted_row_indices): 0467 ¦ reasons.append("row_idx_out_of_range") 0468 ¦ 0469 ¦ valid_in_range = { 0470 ¦ index for index in valid_row_indices if 0 <= index < 480 0471 ¦ } 0472 ¦ expected = set(range(480)) 0473 ¦ missing_rows = tuple(sorted(expected - valid_in_range)) 0474 ¦ max_consecutive = self._max_consecutive(missing_rows) 0475 ¦ if not missing_rows: 0476 ¦ reasons.append("no_missing_rows_to_recover") 0477 ¦ if len(missing_rows) > self.max_missing_rows: 0478 ¦ reasons.append("too_many_missing_rows") 0479 ¦ if max_consecutive >= self.max_consecutive_missing: 0480 ¦ reasons.append("consecutive_missing_rows") 0481 ¦ 0482 ¦ row_lengths = {len(frame.rows[index]) for index in valid_in_range} 0483 ¦ if not row_lengths: 0484 ¦ reasons.append("no_valid_rows") 0485 ¦ elif row_lengths != {ROW_BYTES}: 0486 ¦ reasons.append("row_byte_length_mismatch") 0487 ¦ 0488 ¦ if self.bit_order not in (BitOrder.MSB_FIRST, BitOrder.LSB_FIRST): 0489 ¦ reasons.append("unsupported_pixel_format") 0490 ¦

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\image_pipeline.py:413

表 8-1  三类输出

| 结果 | 条件 | 输出位置 |
| --- | --- | --- |
| COMPLETE | 480行均通过严格校验，不补行 | 正式cam目录 |
| RECOVERED | 可靠LAST、无overflow/sync/CRC/conflict，missing阈值内 | recovered/frame_<id>/ |
| REJECTED | 任一gate失败 | 不生成正式PGM/RAW；记录reason |

--max-consecutive-missing 2｜当前代码把该值作为排除阈值：最大连续缺失 >=2 就拒绝，因此实际只允许单个连续缺行。

# 9. packed 1bpp 到 RAW/PGM

每行Camera payload为80 bytes，代表640个1-bit阈值像素。Image pipeline将每个bit展开为8-bit 0x00/0xFF，形成640×480、307200-byte显示RAW，再加P5 header得到PGM。它不是OV5640原始Y8。

表 9-1  两个image.raw的不同语义

| 路径 | 内容 | 480行大小 |
| --- | --- | --- |
| OutputRoot/cam_<id>/frame_<id>/image.raw | Layer5证据archive，80B packed行 | 38,400 B |
| ImagesRoot/cam<id>/<id>.raw或recovered/... | 展开后的640×480 U8 | 307,200 B |

证据：taxi_receiver\threshold_recover.py:90–148,173–234；taxi_receiver\image_pipeline.py:316–397

# 10. 输出解耦与性能演进

图 10-1  同步写盘热路径与当前异步输出路径

代码对照｜有界frame output queue、阻塞提交和drain/join

0023 ¦ class AsyncCallbackDispatcher(Generic[T]): 0024 ¦ """Run one callback on a dedicated worker with bounded buffering. 0025 ¦ 0026 ¦ ``submit`` blocks only when the bounded output queue is genuinely full. 0027 ¦ It never silently drops a completed frame. This keeps slow atomic disk 0028 ¦ publication out of the packet parsing/reassembly worker while retaining an 0029 ¦ explicit memory limit and an observable queue high-water mark. 0030 ¦ """ 0031 ¦ 0032 ¦ def __init__( 0033 ¦ self, 0034 ¦ callback: Callable[[T], None], 0035 ¦ *, 0036 ¦ queue_depth: int = 256, 0037 ¦ name: str = "taxi-frame-output", 0038 ¦ error_sink: Callable[[str], None] ¦ None = None, 0039 ¦ ) -> None: 0040 ¦ if queue_depth <= 0: 0041 ¦ raise ValueError("queue_depth must be positive") 0042 ¦ self.callback = callback 0043 ¦ self.queue_depth = queue_depth 0044 ¦ self.error_sink = error_sink or ( 0045 ¦ lambda message: print(message, file=sys.stderr) 0046 ¦ ) 0047 ¦ self.stats = AsyncCallbackStatistics() 0048 ¦ self._queue: queue.Queue[T ¦ object] = queue.Queue( 0049 ¦ maxsize=queue_depth 0050 ¦ ) 0051 ¦ self._closed = False 0052 ¦ self._worker = threading.Thread( 0053 ¦ target=self._run, 0054 ¦ name=name, 0055 ¦ daemon=True, 0056 ¦ ) 0057 ¦ self._worker.start() 0058 ¦ 0059 ¦ def submit(self, item: T) -> None: 0060 ¦ if self._closed: 0061 ¦ raise RuntimeError("async callback dispatcher is closed") 0062 ¦ self._queue.put(item) 0063 ¦ self.stats.submitted += 1 0064 ¦ self.stats.queue_peak = max( 0065 ¦ self.stats.queue_peak, 0066 ¦ self._queue.qsize(), 0067 ¦ ) 0068 ¦ 0069 ¦ def close(self) -> None: 0070 ¦ if self._closed: 0071 ¦ return 0072 ¦ self._queue.join() 0073 ¦ self._queue.put(_STOP) 0074 ¦ self._worker.join(timeout=30.0) 0075 ¦ if self._worker.is_alive(): 0076 ¦ raise RuntimeError("async callback worker did not stop")

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\async_sink.py:23

表 10-1  两类queue

| 属性 | Capture packet queue | Frame output queue |
| --- | --- | --- |
| 文件 | pipeline.py | async_sink.py |
| 单位 | RawEthernetFrame | CompletedFrame |
| 生产者 | Npcap/PCAP callback | reassembler |
| 消费者 | taxi-worker | taxi-frame-output |
| 默认深度 | CLI 8192；脚本65536 | 256 |
| 满时 | live显式drop；offline阻塞重试 | 阻塞，不静默丢完整图 |
| 指标 | drops/peak/producer/consumer | submitted/processed/failures/peak |

异步输出把RAW/PGM/JSON/归档移出packet热路径，可吸收短时磁盘抖动；但磁盘长期慢于图像产生速度时，frame output queue仍会满，背压传播到packet worker，再导致capture queue drop。

代码对照｜summary.csv持久追加，避免O(n²)重写

0169 ¦ def _append_summary( 0170 ¦ self, 0171 ¦ frame: CompletedFrame, 0172 ¦ output_path: Path, 0173 ¦ ) -> None: 0174 ¦ row = self._summary_row(frame, output_path) 0175 ¦ with self._summary_lock: 0176 ¦ writer = self._get_summary_writer() 0177 ¦ writer.writerow(row) 0178 ¦ assert self._summary_handle is not None 0179 ¦ self._summary_handle.flush() 0180 ¦ 0181 ¦ def _get_summary_writer(self) -> csv.DictWriter: 0182 ¦ if self._summary_writer is not None: 0183 ¦ return self._summary_writer 0184 ¦ 0185 ¦ has_content = ( 0186 ¦ self.summary_path.exists() 0187 ¦ and self.summary_path.stat().st_size > 0 0188 ¦ ) 0189 ¦ if has_content: 0190 ¦ with self.summary_path.open( 0191 ¦ newline="", encoding="utf-8" 0192 ¦ ) as handle: 0193 ¦ header = next(csv.reader(handle), []) 0194 ¦ if tuple(header) != SUMMARY_FIELDS: 0195 ¦ raise ValueError( 0196 ¦ "existing summary.csv schema does not match current " 0197 ¦ f"receiver: {self.summary_path}" 0198 ¦ ) 0199 ¦ 0200 ¦ self._summary_handle = self.summary_path.open( 0201 ¦ "a", 0202 ¦ newline="", 0203 ¦ encoding="utf-8", 0204 ¦ ) 0205 ¦ self._summary_writer = csv.DictWriter( 0206 ¦ self._summary_handle, 0207 ¦ fieldnames=SUMMARY_FIELDS, 0208 ¦ ) 0209 ¦ if not has_content: 0210 ¦ self._summary_writer.writeheader() 0211 ¦ self._summary_handle.flush() 0212 ¦ return self._summary_writer

证据：D:\prg\prg_cam\prg_cam.srcs\sources_1\tx\taxi_receiver_advanced\taxi_receiver\taxi_receiver\storage.py:169

# 11. Storage、SessionAudit与归档

表 11-1  输出机制

| 组件 | 职责 | 关键边界 |
| --- | --- | --- |
| StorageAndPipeline | temp sibling→文件关闭→os.replace→summary | Windows lock重试 |
| CameraImagePipeline | PGM/显示RAW/JSON/rows.csv | 三文件逐个原子，不是目录事务 |
| SessionAudit | 每包CSV、raw/effective flags、上电回退 | 审计旁路不改变严格校验 |
| analyze_camera_archive | missing、zero-fill、row hash、跨帧复用 | 静止画面必须用hash证明 |

worker异常｜AsyncCallbackDispatcher会记录callback failures并打印，但CLI当前不会自动以非零退出。正式验收必须显式要求output_worker_failures=0。

# 12. 历史演进与因果链

同步接收：Capture后直接处理，功能简单但观测不足。

Layer1–5分层：捕获、Ethernet、Camera解析、监控、重组责任分开。

严格Layer3：sync/payload_len/CRC/overflow/length_error不再静默。

FrameReassembler：按(cam_id,frame_id,row_idx)支持乱序/重复/超时。

RAW/PGM/JSON：区分packed证据与显示图像。

recover-zero-fill：只在严格门槛内对少量missing row写0。

async output：把慢磁盘移出packet热路径。

summary持久追加：从每帧重写历史O(n²)改为接近O(1)。

queue high-water与rate：可以确认瓶颈位于capture还是output。

attempt3因果关系｜877432 - 795297 = 82135；82135 + 2729 = 84864。该轮sequence gaps由capture queue drops和Layer3 length rejection解释，而CRC、固定Ethernet长度、parser errors均为0。

# 13. 测试与真实运行证据

表 13-1  关键测试

| 测试 | 证明 | 不证明 |
| --- | --- | --- |
| test_pipeline_synthetic:慢消费者 | 同步回调drop；异步后100 frame全处理 | 真实Npcap 120秒 |
| test_reassembler | 不同frame_id不复用旧row | 静止场景内容正确 |
| test_image_recovery 15项 | COMPLETE/RECOVERED/REJECTED门槛 | 上游实际缺行率 |
| test_pcap_stdlib | 固定PCAP Layer1/2 | Camera协议 |
| attempt3 analyzer | 1022组PGM/RAW一致、zero-fill位置正确 | 优化后性能 |

表 13-2  attempt3 final report

| 指标 | 值 | 解释 |
| --- | --- | --- |
| Capture ingress | 877432 | Scapy callback可见的入口，不等于NIC kernel统计 |
| Matching Ethernet | 795297 | 进入处理队列的0x88B5 |
| Capture queue drops | 82135 | live队列满主动drop |
| length errors | 2729 | 严格Camera行长度拒绝 |
| complete/recovered | 227 / 795 | usable总1022 |
| total usable fps | 8.453 | 低于约15fps session入口 |

# 14. 当前风险、过期项与PENDING

frame output queue过小吸收不了突发；过大增加内存和退出drain时间。

summary.csv每帧flush但不fsync；优雅退出可刷新，突然断电不等价于落盘保证。

rows.csv/session_audit仍在packet observer路径批量flush，可能成为下一瓶颈。

OutputRoot使用cam_0，ImagesRoot使用cam0，培训和脚本中必须区分。

固定PCAP位于被.gitignore忽略的build目录，clean clone不自动拥有。

表 14-1  过期结论

| 旧表述 | 当前事实 |
| --- | --- |
| 只有COMPLETE才发布 | 显式recover-zero-fill可输出RECOVERED |
| 0x04是overflow | 0x01 overflow；0x04 FIRST_ROW |
| 正式sync A5A5/5A5A | 当前A5A0/5A50 |
| 27/37/51项测试 | 当前实测88项 |
| image.raw都是同一种格式 | packed archive与U8 display RAW不同 |

PENDING｜性能优化后的同条件120秒live final report、当前bit/固件/PCAP hash绑定、真实场景图像正确性和Npcap kernel/NIC drop统计仍未确认。